TEMPLATE — FILL FIELDS SHOWN IN RED. REVIEW WITH LEGAL COUNSEL BEFORE EXECUTION.

AMERICANS TRADES GROUP LLC

Tire-Derived Fuel · Surplus Tire Program

PRODUCT SPECIFICATION SHEET

Tyre-Derived Fuel (TDF) for Cement Kiln Co-Processing

Document Ref: ATG-TDF-SPEC-2026-v1.0    |    Issue Date: ____ ____________ 2026    |    Supersedes: [ — ]

1..  Product Description

Used pneumatic tyres (whole or shredded) of rubber, originating from passenger vehicles, light commercial vehicles, heavy goods vehicles, off-the-road and earth-moving applications, intended for co-processing as alternative thermal fuel in cement kilns. Material shall be classified as non-hazardous waste under the Basel Convention, Annex IX, Entry B3140.

2..  Identification

| Trade Name | Tyre-Derived Fuel (TDF) |
| --- | --- |
| HS Code / PCT | 4012.20.00 — Used pneumatic tyres of rubber |
| Basel Classification | Annex IX, Entry B3140 (non-hazardous) |
| UN Number | Not regulated as dangerous goods (UN — n/a) |
| CAS Number | Mixture (rubber, carbon black, steel cord, textile) |
| Country of Origin | [Brazil / Chile / Argentina / Peru / Colombia / Uruguay] |

3..  Physical Form & Sizing

| Format A — Whole tyres | Loaded loose into containers; suitable for precalciner injection. Tyres must be free of rims, wheel weights and water. |
| --- | --- |
| Format B — Shredded TDF | Mechanically shredded to nominal 50–150 mm chips; suitable for main-burner injection. Maximum 5% (m/m) of chips outside the 50–150 mm range. |
| Steel content (Format B) | Steel wire content as-is (uncleaned) acceptable, unless cement plant specifies de-wired TDF (subject to separate spec). |
| Foreign matter | ≤ 1% (m/m) of soil, stones, plastics or other non-rubber matter. No free liquids, no asbestos, no radioactive material. |
| Loading | ≈ 22 MT per 40-ft HC (whole) or ≈ 25 MT per 40-ft HC (shredded). Each container photographed before sealing. |

4..  Quality Parameters (As-Delivered Basis)

| Parameter | Specification |
| --- | --- |
| Net Calorific Value (NCV / LHV) | 28 – 32 MJ/kg (minimum 28 MJ/kg) |
| Moisture Content | ≤ 2.0% (m/m) |
| Ash Content | ≤ 22% (m/m) |
| Volatile Matter | 55 – 65% (m/m) |
| Fixed Carbon | 20 – 30% (m/m) |
| Total Sulfur (S) | ≤ 2.0% (m/m) |
| Total Chlorine (Cl) | ≤ 0.5% (m/m) |
| Mercury (Hg) | ≤ 0.5 mg/kg |
| Cadmium (Cd) | ≤ 5 mg/kg |
| Lead (Pb) | ≤ 200 mg/kg |
| Chromium (Cr — total) | ≤ 200 mg/kg |
| Bulk Density (whole) | ≈ 350 – 450 kg/m³ |
| Bulk Density (shred) | ≈ 450 – 600 kg/m³ |

All values determined on as-delivered basis using the following reference methods (or equivalent): NCV per ASTM D5865 / ISO 1928; Moisture per ASTM D3173 / ISO 11722; Ash per ASTM D3174 / ISO 1171; Sulphur per ASTM D4239 / ISO 19579; Chlorine per ASTM D6721 / ISO 587; metals by ICP-MS / ICP-OES after acid digestion.

5..  Sampling, Testing & Pre-Shipment Inspection

1.  Sampling shall be performed by an internationally recognised independent surveyor (SGS, Bureau Veritas, Intertek, Cotecna or equivalent) at the consolidation yard prior to container stuffing.

2.  A composite sample shall be drawn from each lot of up to 200 MT, split into three (3) sealed sub-samples: one for the laboratory, one for the Buyer, one retained by the surveyor for ninety (90) days.

3.  A Pre-Shipment Inspection (PSI) Certificate and a Certificate of Analysis (CoA) shall accompany every consignment.

4.  The Buyer or its appointed laboratory may, at its discretion, re-test on arrival. In case of variance > 10% on any parameter, the umpire analysis convention shall apply (independent third laboratory mutually agreed, whose result shall be final and binding).

6..  Packaging, Loading & Documentation

| Packaging | Loose stuffing in dry, clean 40-ft High-Cube containers. No bagging or pallets required. |
| --- | --- |
| Seals | Bolt seal + cable seal applied at point of stuffing; numbers logged and shared with Buyer prior to vessel sailing. |
| Photo log | Minimum 8 photos per container: empty, half-loaded, fully loaded, doors closing, both seals affixed. |
| Vector control | Mandatory written declaration that all material is dry and free of vector-attracting matter (Aedes aegypti / mosquito control). |
| Phytosanitary | ISPM-15 fumigation certificate required if any wood pallets or dunnage used. |
| Documentation set | Per shipment: Commercial Invoice, Packing List, BL, Certificate of Origin, Marine Insurance Certificate, PSI Certificate, CoA, Basel Annex IB. |

7..  Rejection Criteria

1.  Moisture > 3.0% on arrival (after deducting transit gain).

2.  Net calorific value < 26 MJ/kg on arrival.

3.  Chlorine > 0.7% or Sulfur > 2.5% on arrival.

4.  Presence of any radioactive, asbestos-containing or hazardous waste material.

5.  Foreign matter > 3% (m/m) by weight.

6.  Tampered seals or undocumented chain of custody.

7.  Material not corresponding to the agreed format (whole vs shred) or outside the agreed sizing tolerance.

8..  Regulatory References

1.  Basel Convention on the Control of Transboundary Movements of Hazardous Wastes and their Disposal (1989), Annex IX entry B3140.

2.  Pakistan Environmental Protection Act, 1997 and Hazardous Substances Rules, 2003.

3.  Pakistan National Environmental Quality Standards (NEQS) for co-processing emissions.

4.  Cement Sustainability Initiative (CSI / GCCA) Guidelines for Co-Processing Fuels and Raw Materials in Cement Manufacturing.

5.  ISO 21640:2021 — Solid recovered fuels — Specifications and classes (reference framework for SRF/TDF parameters).

9..  Approvals

| Buyer: AMERICANS TRADES GROUP LLC By: ____________________________ Name: ____________________________ Title: ____________________________ Date: ____________________________ | Supplier: [SUPPLIER LEGAL NAME] By: ____________________________ Name: ____________________________ Title: ____________________________ Date: ____________________________ |
| --- | --- |

This is a template document prepared for internal use. It is not legal advice. Engage qualified counsel in the relevant jurisdiction(s) before executing any binding agreement.